IoT Environmental Monitoring System using Arduino UNO R4 WiFi, DHT11, MQ135, and LCD Display

## 1. Objective

The objective of this project is to design and implement an embedded IoT system capable of:

- Reading temperature, humidity, and air quality using DHT11 and MQ135 sensors.

- Displaying real-time sensor data locally on a 16×2 LCD via I²C.

- Transmitting the data to a public MQTT broker for remote monitoring.

- Receiving remote MQTT commands to control hardware (LED) and LCD interface.

This project demonstrates the integration of sensing, display, and IoT communication—fulfilling the requirements of the Antariksha Labs Embedded Assignment.

## 2. System Components

| Component | Description | Function |
| --- | --- | --- |
| Arduino UNO R4 WiFi | Main microcontroller with built-in Wi-Fi and MQTT capability | Controls sensors, display, and IoT communication |
| DHT11 Sensor | Temperature & humidity sensor | Provides ambient environmental data |
| MQ135 Gas Sensor | Air quality sensor (analog) | Measures pollution or air quality levels |
| LCD 16×2 (I²C) | Display module | Shows temperature, humidity, and air values |
| LED | Output indicator | Responds to MQTT commands |
| Wi-Fi Network | Internet connection | Enables MQTT communication |
| MQTT Broker | test.mosquitto.org | Publishes sensor data and receives commands |

## 3. System Architecture

### Data Flow:

- Sensors (DHT11 & MQ135) continuously measure temperature, humidity, and air quality.

- Arduino UNO R4 WiFi reads sensor data, processes it, and updates the LCD display every 2 seconds.

- The data is converted to JSON format and published to the MQTT topic iot/env_monitor.

- The MCU subscribes to the topic iot/env_monitor/cmd to receive control commands.

- Remote commands like LED_ON, LED_OFF, or CLR are executed accordingly.

## 4. MQTT Communication

| Direction | Topic | Example Payload | Function |
| --- | --- | --- | --- |
| Publish | iot/env_monitor | {"temp":27,"hum":61,"air":340} | Sends sensor readings to broker |

Broker Used: test.mosquitto.org (port 1883 for Arduino, port 8081 for WebSocket client)

5. Circuit Connections

| Component | Arduino UNO R4 WiFi Pin | Description |
| --- | --- | --- |
| DHT11 | D4 | Data pin |
| MQ135 | A0 | Analog output |
| LCD (I²C) | A4 (SDA), A5 (SCL) | I²C communication |
| LED | D13 | Status indicator |
| All GND pins | Common ground | Shared ground reference |
| Power | 5V | Powers sensors and LCD |

## 6. Results & Observations

- The LCD displays real-time temperature (°C), humidity (%), and air quality index.

- MQTT data successfully published and viewed on the HiveMQ/Mosquitto WebSocket client.

- Commands like LED_ON, LED_OFF, and CLR remotely control the device.

- Reliable Wi-Fi connectivity and stable MQTT transmission every 2 seconds.

## 7. Challenges Faced

- Implementing manual DHT11 driver for raw bit-level data reading.

- Debugging I²C LCD address mismatch (0x27 vs 0x3F).

- Handling Wi-Fi connection timeouts gracefully to prevent system hang.

- Ensuring stable MQTT connectivity with Mosquitto public broker.

## 8. Conclusion

This project successfully demonstrates a complete embedded IoT system that integrates:

- Low-level sensor data acquisition

- Real-time local visualization

- MQTT-based cloud communication and remote control

It effectively showcases embedded firmware development, networking, and system integration skills — aligning perfectly with the Antariksha Labs Embedded Mini Project requirements.

## 9. References

- Arduino UNO R4 WiFi Documentation — https://docs.arduino.cc

- HiveMQ Public MQTT Broker — https://www.hivemq.com/demos/websocket-client/

- Eclipse Mosquitto MQTT Broker — https://test.mosquitto.org